Grainfather Brewing Recipes # 5

Copper Kettle - Golden Ale

This ale was made directly from an all grain kit purchased online from Copper Kettle Home Brewing.

| Ingredients | - Maris Otter Pale Malt (4.6 kg) - Challenger Bittering Hops (47g) - Golding Flavouring Hops (16g) - Golding Aroma Hops (10g) - Nottingham Ale Yeast (11.5g) - 1/2 Protofloc tablet |
| --- | --- |
| Mashing (90 min) | - Add 16l of water to Grainfather and heat to 66oC - Add the grain slowly whilst stirring - to avoid clumping - Start pump and mash for 90 minutes |
| Sparging | - After 60 minutes of mash prepare 17l of water @ 78oC - Raise Grainfather temperature from 65oC to 78oC - When 90 minute mash is complete raise inner basket - Sparge steadily to achieve 28l of wort - Leave to drain for 10 minutes, setting grainfather to boil. - Once sparge is complete remove inner basket |
| Boiling (90 min) | - When temperature control beeps and "HH" displayed, add Challenger (47g) bittering hops in hop sock (0 min) - Stir carefully to reduce foam and prevent boiling over - After 75 minutes add 1/2 protofloc tablet (75 min) - After 80 minutes add Golding (16g) flavouring hops in hop sock (80 min) - At flame out (90 min) cool to 80oC then add Golding (10g) Aroma hops - Rest the wort for 30 minutes before cooling (steep aroma hops) |
| Cooling & Racking | - Set up wort chiller while wort cools - Cool wort to 20oC using recirculation pump - Measure and record Specific Gravity (OG) |
| Fermenting & Racking | - Pitch the dry yeast across the surface of the brew - Ferment for 7 days until complete maintaining temperature at 22oC using brew box - Measure and record Specific Gravity (FG) - Rack again and after a further 7 days keg |
| Kegging 1 Cornelius | - Rack into Cornelius keg - Connect CO2 and pressurize to 25 psi, rock keg gently - Reduce pressure to 12 psi and leave for 7 days - Reduce pressure to 2 psi attach tap and DRINK. |
| Bottling 3 Bottles | - Add golden caster sugar 5g/l to each bottle - Rack, leaving approx 1/2" at top of bottles cap bottles - Swirl sugar until dissolved. - Store at room temperature for 7 days - Store at approx 10oC for further 7 days and DRINK. |

Calculations:

1.	Mash water (Litres):   	VMash Water  = (Grain Weight in kgs x 2.7) + 3.5

VMash Water  = (4.6 x 2.7) + 3.5 = 15.9 = 16 Litres

2.	Sparge water (Litres):	VMash Water  = (29 - VMash Water) + (Grain Weight x 0.8)

VMash Water  = (29 - 16) + (4.8 x 0.8) = 16.8 Litres

3.	Alcohol:			ABV = (SGInitial - SGFinal) x 131

ABV = (1.045 - 1.012) x 131 = 4.3%

Records:

| Brew Date | 19/11/2016 |
| --- | --- |
| Initial Specific Gravity | 1045 |
| Final Specific gravity | 1012 |
| Alcohol | 4.3% |
| Bitterness (IBU) | 55 |
| Colour | EBC 10.3, SRM 5.3. |
| Aroma |  |
| Taste |  |
| Other |  |